LL RS485 Grind controller instructions sheet
USE AT YOUR OWN RISK, THIS IS A DEV PROJECT NOT A PRODUCT

INSTALL

Tbd

FIRST STARTUP
The controller is loaded up with default values which have been verified on a RT version of the LL SG98. Please be mindful changing settings as they have effect on the way the servo drive is instructed.

Please verify the controller works and the grinder spins up in the correct direction when pressing the START button while connected and in NORMAL mode (not GBW mode).

Then in Menu1, find “Calibrate” and complete the procedure by long pressing the SELECT button to start the process. It will stop by itself.

IDLE

In IDLE, the display will show the set RPM, connection status and

GRINDING

The controller has two basic functions: NORMAL and GBW. In normal mode, the controller will detect the grinding of beans and will purge or stop the motor automatically (can be changed in settings).

After 60s of running without detecting grinding or button interactions, the grinder will stop. If you press the SELECT button the motor will stop. When purge w/ button is ENABLED, START will continue to purge, otherwise it will also stop the motor.

Long pressing the SELECT button will change to GBW mode

PURGING

(RUNNING->GRINDING->(delay)->STOP->FORWARD->BACKWARD)

Grinding is detected based on the motor having to work harder. You can disable auto purge, change the delay or duration, change the purge RPM (0 to disable purge & only auto stop) and add a reverse purge by setting the RPM higher than 0. (Default: 0). You can also enable purging by pressing the START button, if set up in settings.

GBW:

Long pressing the SELECT button will change to NORMAL mode (and disconnect the scale)

Note: The resulting ground coffee is slightly finer in this mode.

You need a scale for this, recommend using the BOOKOO THEMIS MINI (only Themis mini + bookoo themis mini verified to work!).

After connecting the scale (automatically connects to a scale which has Bluetooth enabled), place it under the output dosing cup. Fill the ‘hopper’ with beans. Select the weight required. Press START to start the GBW grinding. After it has finished, the scale will beep. 
The controller will self-learn and become more accurate with use.

Menu

Press the SELECT button to enter the menu while in IDLE or IDLE_GBW. Press SELECT to continue inside a submenu or a setting. Pressing SELECT while in a setting SAVES the set value. If you press START while in the menu, you go back, without saving anything.

### Menu1

- EXIT MENU

- Brightness – sets the display brightness level

- Purge Settings – enters the purge settings submenu

- GbW Settings – enters the gbw settings submenu

- Calibrate – calibration for auto purge

- Max RPM – sets the max rpm

- Min RPM – sets the min rpm

- Sleep time – time in minutes after which the controller will sleep. Set 0 to never sleep.

- Motor torque 0.1% - sets the motor torque. Default 1040 for RT and xx for JMC

- LED max brightness – A scalar for the button LED. Setting to 0 will disable the light.

- Invert scrolling – this will invert the menu scrolling. (Changing values will still be CW positive)

### Menu2

- BACK

- Purge /w button – Enabled purging by pressing the START button while grinding

- Purge frames LOW – Auto purge detection setting <500rpm

- Purge frames HIGH – Auto purge detection setting >500rpm

- Purge Scalar LOW – Auto purge detection setting  <500rpm

- Purge Scalar HIGH – Auto purge detection setting >500rpm

- Purge duration – The time each purge phase will last (high rpm time)

- Purge delay – The time between grinding stopped and purge commence

- Reverse rotation – Reverse purge RPM (0 to disable)

- Auto purge enabled – Enable auto purge detection

- Purge stabilize time – Auto purge detection setting

- Purge forward RPM – Forward purge RPM (0 to disable)

- Purge 0rpm time – The time the motor will be off before purging (0 to instantly ramp up)

Menu3

- BACK

- GbW RPM (RPM used for GbW grinding)

- GbW Slow Phase t (how long the slow phase grinding will last)

- GbW Slow RPM (the RPM during slow phase grinding.. to creep up on target weight)

- SpeedModifier (Auto learning value, will self adjust)

- Start delay (the time between pressing the button and start, to prevent button press wobble causing wrong readings)

- Time offset (auto learning value, will self adjust)

- Save scale (only connect to the last connected scale in the future, so the controller does not hijack other scales)